CÉSAR KALLAS

FERNANDO HAFNER

REDES DE COMPUTADORES iI

Relatório do Laboratório I

Pontifícia Universidade Católica de Campinas

Faculdade de Engenharia dE Computação

Abril /2006

César Kallas		R.A. 02099224	email: cesarkallas@gmx.net

Fernando Hafner	R.A. 01056209	email: fernando.hafner@gmail.com

Divisão das Notas:

César Kallas	-	60%

Fernando Hafner	-	40%

LAB1 / LIEBEHERR

Trabalho requisitado pelo professor de Redes de Computadores II, ministrada pelo professor Juan, da Pontifícia Universidade Católica, com o intuito de avaliação acadêmica dos alunos, para efeito de cálculo da média final.

Pontifícia Universidade Católica de Campinas

Faculdade de Engenharia dE Computação

Abril /2006

1. ) Pré-Lab

  - ) Respostas do Pré-Lab

Exercício 5

Lab-report:

Para mudar o nome da máquina, editar o arquivo /etc/hostname

roteamento de ip: /etc/sysctl.conf

Exercício 6

Diferença de ping na internet eth0 e no loopback ?

Se outro computador receber um ping, o pacote irá trafegar pela interface de rede eth0, pela camada fisica e pelo roteador/hub/switch.

Se o ping for feito na interface de loopback, o ping não irá sair do computador, porque é uma interface de rede virtual da máquina, não tendo placa física.

Exercício 7 (b):

Todos os 4 computadores respoderam ao ping

192.168.0.1 (ele mesmo)

192.168.0.2

192.168.0.3

192.168.0.4

Exercício 8:

Tcpdump: Mostra por onde o tráfego da rede está passando

Ethereal: Mosta por onde o tráfego da rede está passando, incluindo o tráfego (os dados) 	e o cabecalho dos pacotes, exibe mais detalhes.

1.2) Respostas às perguntas do Pré-Lab

1-What will happen if you type man man in linux?

Exibe as informações sobre o comando MAN

2- How can you used the command ls to find out about the size of file /etc/lilo.conf?

LS- L exibe detalhes dos arquivos que estão no diretório local

3- What happens if you have two files with names file1 and file2 and you type mv file1 file2? Which option of mv issue a warning in this situation?

Copia o conteúdo do file1 para o file2 e remove o file1

4- What is the command that you issue if you are in directory / and want to copy the file /mydata to directory /labdata?

cp mydata labdata

5- What is the command that you issue if you are in directory / and want to copy all files and directories under directories /mydirectory to directory /newdirectory?

cp –r  my directory newdirectory

Cria de maneira recursiva todos arquivos e sub-diretórios do directory para o newdirectory.

6 – What happens if you type the command rm * in a directory?

Remove todos os arquivos do diretório corrente. Mantém o sub-diretórios inalterados.

7- What is the command that you issue if you want delete all files and directories under the directory /mydirectory?

rm –r

Remove de maneira recursiva todos arquivos e sub-diretórios do diretório corrente.

2. ) Análise de tráfego com o Ethereal

No.     Time        Source                Destination           Protocol Info

1 0.000000    00:08:74:b5:c8:b7     ff:ff:ff:ff:ff:ff     ARP      Who has 192.168.0.3?  Tell 192.168.0.1

Frame 1 (42 bytes on wire, 42 bytes captured)

Arrival Time: Apr 17, 2006 21:00:35.426303000

Time delta from previous packet: 0.000000000 seconds

Time since reference or first frame: 0.000000000 seconds

Frame Number: 1

Packet Length: 42 bytes

Capture Length: 42 bytes

Protocols in frame: eth:arp

Ethernet II, Src: 00:08:74:b5:c8:b7 (00:08:74:b5:c8:b7), Dst: ff:ff:ff:ff:ff:ff (ff:ff:ff:ff:ff:ff)

Destination: ff:ff:ff:ff:ff:ff (ff:ff:ff:ff:ff:ff)

Source: 00:08:74:b5:c8:b7 (00:08:74:b5:c8:b7)

Type: ARP (0x0806)

Address Resolution Protocol (request)

Hardware type: Ethernet (0x0001)

Protocol type: IP (0x0800)

Hardware size: 6

Protocol size: 4

Opcode: request (0x0001)

Sender MAC address: 00:08:74:b5:c8:b7 (00:08:74:b5:c8:b7)

Sender IP address: 192.168.0.1 (192.168.0.1)

Target MAC address: 00:00:00:00:00:00 (00:00:00:00:00:00)

Target IP address: 192.168.0.3 (192.168.0.3)

No.     Time        Source                Destination           Protocol Info

2 0.000305    00:e0:4c:78:c0:a6     00:08:74:b5:c8:b7     ARP      192.168.0.3 is at 00:e0:4c:78:c0:a6

Frame 2 (60 bytes on wire, 60 bytes captured)

Arrival Time: Apr 17, 2006 21:00:35.426608000

Time delta from previous packet: 0.000305000 seconds

Time since reference or first frame: 0.000305000 seconds

Frame Number: 2

Packet Length: 60 bytes

Capture Length: 60 bytes

Protocols in frame: eth:arp

Ethernet II, Src: 00:e0:4c:78:c0:a6 (00:e0:4c:78:c0:a6), Dst: 00:08:74:b5:c8:b7 (00:08:74:b5:c8:b7)

Destination: 00:08:74:b5:c8:b7 (00:08:74:b5:c8:b7)

Source: 00:e0:4c:78:c0:a6 (00:e0:4c:78:c0:a6)

Type: ARP (0x0806)

Trailer: 000000000000000000000000000000000000

Address Resolution Protocol (reply)

Hardware type: Ethernet (0x0001)

Protocol type: IP (0x0800)

Hardware size: 6

Protocol size: 4

Opcode: reply (0x0002)

Sender MAC address: 00:e0:4c:78:c0:a6 (00:e0:4c:78:c0:a6)

Sender IP address: 192.168.0.3 (192.168.0.3)

Target MAC address: 00:08:74:b5:c8:b7 (00:08:74:b5:c8:b7)

Target IP address: 192.168.0.1 (192.168.0.1)

No.     Time        Source                Destination           Protocol Info

3 0.000319    192.168.0.1           192.168.0.3           ICMP     Echo (ping) request

Frame 3 (98 bytes on wire, 98 bytes captured)

Arrival Time: Apr 17, 2006 21:00:35.426622000

Time delta from previous packet: 0.000014000 seconds

Time since reference or first frame: 0.000319000 seconds

Frame Number: 3

Packet Length: 98 bytes

Capture Length: 98 bytes

Protocols in frame: eth:ip:icmp:data

Ethernet II, Src: 00:08:74:b5:c8:b7 (00:08:74:b5:c8:b7), Dst: 00:e0:4c:78:c0:a6 (00:e0:4c:78:c0:a6)

Destination: 00:e0:4c:78:c0:a6 (00:e0:4c:78:c0:a6)

Source: 00:08:74:b5:c8:b7 (00:08:74:b5:c8:b7)

Type: IP (0x0800)

Internet Protocol, Src: 192.168.0.1 (192.168.0.1), Dst: 192.168.0.3 (192.168.0.3)

Version: 4

Header length: 20 bytes

Differentiated Services Field: 0x00 (DSCP 0x00: Default; ECN: 0x00)

0000 00.. = Differentiated Services Codepoint: Default (0x00)

.... ..0. = ECN-Capable Transport (ECT): 0

.... ...0 = ECN-CE: 0

Total Length: 84

Identification: 0x0000 (0)

Flags: 0x04 (Don't Fragment)

0... = Reserved bit: Not set

.1.. = Don't fragment: Set

..0. = More fragments: Not set

Fragment offset: 0

Time to live: 64

Protocol: ICMP (0x01)

Header checksum: 0xb954 [correct]

Source: 192.168.0.1 (192.168.0.1)

Destination: 192.168.0.3 (192.168.0.3)

Internet Control Message Protocol

Type: 8 (Echo (ping) request)

Code: 0

Checksum: 0x32dd [correct]

Identifier: 0x7530

Sequence number: 0x0001

Data (56 bytes)

0000  23 2c 44 44 f7 7d 06 00 08 09 0a 0b 0c 0d 0e 0f   #,DD.}..........

0010  10 11 12 13 14 15 16 17 18 19 1a 1b 1c 1d 1e 1f   ................

0020  20 21 22 23 24 25 26 27 28 29 2a 2b 2c 2d 2e 2f    !"#$%&'()*+,-./

0030  30 31 32 33 34 35 36 37                           01234567

No.     Time        Source                Destination           Protocol Info

4 0.000680    192.168.0.3           192.168.0.1           ICMP     Echo (ping) reply

Frame 4 (98 bytes on wire, 98 bytes captured)

Arrival Time: Apr 17, 2006 21:00:35.426983000

Time delta from previous packet: 0.000361000 seconds

Time since reference or first frame: 0.000680000 seconds

Frame Number: 4

Packet Length: 98 bytes

Capture Length: 98 bytes

Protocols in frame: eth:ip:icmp:data

Ethernet II, Src: 00:e0:4c:78:c0:a6 (00:e0:4c:78:c0:a6), Dst: 00:08:74:b5:c8:b7 (00:08:74:b5:c8:b7)

Destination: 00:08:74:b5:c8:b7 (00:08:74:b5:c8:b7)

Source: 00:e0:4c:78:c0:a6 (00:e0:4c:78:c0:a6)

Type: IP (0x0800)

Internet Protocol, Src: 192.168.0.3 (192.168.0.3), Dst: 192.168.0.1 (192.168.0.1)

Version: 4

Header length: 20 bytes

Differentiated Services Field: 0x00 (DSCP 0x00: Default; ECN: 0x00)

0000 00.. = Differentiated Services Codepoint: Default (0x00)

.... ..0. = ECN-Capable Transport (ECT): 0

.... ...0 = ECN-CE: 0

Total Length: 84

Identification: 0x9e77 (40567)

Flags: 0x00

0... = Reserved bit: Not set

.0.. = Don't fragment: Not set

..0. = More fragments: Not set

Fragment offset: 0

Time to live: 64

Protocol: ICMP (0x01)

Header checksum: 0x5add [correct]

Source: 192.168.0.3 (192.168.0.3)

Destination: 192.168.0.1 (192.168.0.1)

Internet Control Message Protocol

Type: 0 (Echo (ping) reply)

Code: 0

Checksum: 0x3add [correct]

Identifier: 0x7530

Sequence number: 0x0001

Data (56 bytes)

0000  23 2c 44 44 f7 7d 06 00 08 09 0a 0b 0c 0d 0e 0f   #,DD.}..........

0010  10 11 12 13 14 15 16 17 18 19 1a 1b 1c 1d 1e 1f   ................

0020  20 21 22 23 24 25 26 27 28 29 2a 2b 2c 2d 2e 2f    !"#$%&'()*+,-./

0030  30 31 32 33 34 35 36 37                           01234567

No.     Time        Source                Destination           Protocol Info

5 0.999853    192.168.0.1           192.168.0.3           ICMP     Echo (ping) request

Frame 5 (98 bytes on wire, 98 bytes captured)

Arrival Time: Apr 17, 2006 21:00:36.426156000

Time delta from previous packet: 0.999173000 seconds

Time since reference or first frame: 0.999853000 seconds

Frame Number: 5

Packet Length: 98 bytes

Capture Length: 98 bytes

Protocols in frame: eth:ip:icmp:data

Ethernet II, Src: 00:08:74:b5:c8:b7 (00:08:74:b5:c8:b7), Dst: 00:e0:4c:78:c0:a6 (00:e0:4c:78:c0:a6)

Destination: 00:e0:4c:78:c0:a6 (00:e0:4c:78:c0:a6)

Source: 00:08:74:b5:c8:b7 (00:08:74:b5:c8:b7)

Type: IP (0x0800)

Internet Protocol, Src: 192.168.0.1 (192.168.0.1), Dst: 192.168.0.3 (192.168.0.3)

Version: 4

Header length: 20 bytes

Differentiated Services Field: 0x00 (DSCP 0x00: Default; ECN: 0x00)

0000 00.. = Differentiated Services Codepoint: Default (0x00)

.... ..0. = ECN-Capable Transport (ECT): 0

.... ...0 = ECN-CE: 0

Total Length: 84

Identification: 0x0001 (1)

Flags: 0x04 (Don't Fragment)

0... = Reserved bit: Not set

.1.. = Don't fragment: Set

..0. = More fragments: Not set

Fragment offset: 0

Time to live: 64

Protocol: ICMP (0x01)

Header checksum: 0xb953 [correct]

Source: 192.168.0.1 (192.168.0.1)

Destination: 192.168.0.3 (192.168.0.3)

Internet Control Message Protocol

Type: 8 (Echo (ping) request)

Code: 0

Checksum: 0x96d9 [correct]

Identifier: 0x7530

Sequence number: 0x0002

Data (56 bytes)

0000  24 2c 44 44 92 80 06 00 08 09 0a 0b 0c 0d 0e 0f   $,DD............

0010  10 11 12 13 14 15 16 17 18 19 1a 1b 1c 1d 1e 1f   ................

0020  20 21 22 23 24 25 26 27 28 29 2a 2b 2c 2d 2e 2f    !"#$%&'()*+,-./

0030  30 31 32 33 34 35 36 37                           01234567

No.     Time        Source                Destination           Protocol Info

6 1.000199    192.168.0.3           192.168.0.1           ICMP     Echo (ping) reply

Frame 6 (98 bytes on wire, 98 bytes captured)

Arrival Time: Apr 17, 2006 21:00:36.426502000

Time delta from previous packet: 0.000346000 seconds

Time since reference or first frame: 1.000199000 seconds

Frame Number: 6

Packet Length: 98 bytes

Capture Length: 98 bytes

Protocols in frame: eth:ip:icmp:data

Ethernet II, Src: 00:e0:4c:78:c0:a6 (00:e0:4c:78:c0:a6), Dst: 00:08:74:b5:c8:b7 (00:08:74:b5:c8:b7)

Destination: 00:08:74:b5:c8:b7 (00:08:74:b5:c8:b7)

Source: 00:e0:4c:78:c0:a6 (00:e0:4c:78:c0:a6)

Type: IP (0x0800)

Internet Protocol, Src: 192.168.0.3 (192.168.0.3), Dst: 192.168.0.1 (192.168.0.1)

Version: 4

Header length: 20 bytes

Differentiated Services Field: 0x00 (DSCP 0x00: Default; ECN: 0x00)

0000 00.. = Differentiated Services Codepoint: Default (0x00)

.... ..0. = ECN-Capable Transport (ECT): 0

.... ...0 = ECN-CE: 0

Total Length: 84

Identification: 0x9e78 (40568)

Flags: 0x00

0... = Reserved bit: Not set

.0.. = Don't fragment: Not set

..0. = More fragments: Not set

Fragment offset: 0

Time to live: 64

Protocol: ICMP (0x01)

Header checksum: 0x5adc [correct]

Source: 192.168.0.3 (192.168.0.3)

Destination: 192.168.0.1 (192.168.0.1)

Internet Control Message Protocol

Type: 0 (Echo (ping) reply)

Code: 0

Checksum: 0x9ed9 [correct]

Identifier: 0x7530

Sequence number: 0x0002

Data (56 bytes)

0000  24 2c 44 44 92 80 06 00 08 09 0a 0b 0c 0d 0e 0f   $,DD............

0010  10 11 12 13 14 15 16 17 18 19 1a 1b 1c 1d 1e 1f   ................

0020  20 21 22 23 24 25 26 27 28 29 2a 2b 2c 2d 2e 2f    !"#$%&'()*+,-./

0030  30 31 32 33 34 35 36 37                           01234567

No.     Time        Source                Destination           Protocol Info

7 4.999280    00:e0:4c:78:c0:a6     00:08:74:b5:c8:b7     ARP      Who has 192.168.0.1?  Tell 192.168.0.3

Frame 7 (60 bytes on wire, 60 bytes captured)

Arrival Time: Apr 17, 2006 21:00:40.425583000

Time delta from previous packet: 3.999081000 seconds

Time since reference or first frame: 4.999280000 seconds

Frame Number: 7

Packet Length: 60 bytes

Capture Length: 60 bytes

Protocols in frame: eth:arp

Ethernet II, Src: 00:e0:4c:78:c0:a6 (00:e0:4c:78:c0:a6), Dst: 00:08:74:b5:c8:b7 (00:08:74:b5:c8:b7)

Destination: 00:08:74:b5:c8:b7 (00:08:74:b5:c8:b7)

Source: 00:e0:4c:78:c0:a6 (00:e0:4c:78:c0:a6)

Type: ARP (0x0806)

Trailer: 000000000000000000000000000000000000

Address Resolution Protocol (request)

Hardware type: Ethernet (0x0001)

Protocol type: IP (0x0800)

Hardware size: 6

Protocol size: 4

Opcode: request (0x0001)

Sender MAC address: 00:e0:4c:78:c0:a6 (00:e0:4c:78:c0:a6)

Sender IP address: 192.168.0.3 (192.168.0.3)

Target MAC address: 00:00:00:00:00:00 (00:00:00:00:00:00)

Target IP address: 192.168.0.1 (192.168.0.1)

No.     Time        Source                Destination           Protocol Info

8 4.999310    00:08:74:b5:c8:b7     00:e0:4c:78:c0:a6     ARP      192.168.0.1 is at 00:08:74:b5:c8:b7

Frame 8 (42 bytes on wire, 42 bytes captured)

Arrival Time: Apr 17, 2006 21:00:40.425613000

Time delta from previous packet: 0.000030000 seconds

Time since reference or first frame: 4.999310000 seconds

Frame Number: 8

Packet Length: 42 bytes

Capture Length: 42 bytes

Protocols in frame: eth:arp

Ethernet II, Src: 00:08:74:b5:c8:b7 (00:08:74:b5:c8:b7), Dst: 00:e0:4c:78:c0:a6 (00:e0:4c:78:c0:a6)

Destination: 00:e0:4c:78:c0:a6 (00:e0:4c:78:c0:a6)

Source: 00:08:74:b5:c8:b7 (00:08:74:b5:c8:b7)

Type: ARP (0x0806)

Address Resolution Protocol (reply)

Hardware type: Ethernet (0x0001)

Protocol type: IP (0x0800)

Hardware size: 6

Protocol size: 4

Opcode: reply (0x0002)

Sender MAC address: 00:08:74:b5:c8:b7 (00:08:74:b5:c8:b7)

Sender IP address: 192.168.0.1 (192.168.0.1)

Target MAC address: 00:e0:4c:78:c0:a6 (00:e0:4c:78:c0:a6)

Target IP address: 192.168.0.3 (192.168.0.3)

3. ) Ping de 192.168.0.1 para 192.168.0.2

PING 192.168.0.2 (192.168.0.2) 56(84) bytes of data.

64 bytes from 192.168.0.2: icmp_seq=1 ttl=64 time=1.22 ms

64 bytes from 192.168.0.2: icmp_seq=2 ttl=64 time=0.360 ms

64 bytes from 192.168.0.2: icmp_seq=3 ttl=64 time=0.300 ms

64 bytes from 192.168.0.2: icmp_seq=4 ttl=64 time=0.361 ms

64 bytes from 192.168.0.2: icmp_seq=5 ttl=64 time=0.303 ms

--- 192.168.0.2 ping statistics ---

5 packets transmitted, 5 received, 0% packet loss, time 4004ms

rtt min/avg/max/mdev = 0.300/0.509/1.223/0.358 ms

4. ) TCPDump de 192.168.0.1 para 192.168.0.2 utilizando ping

tcpdump: verbose output suppressed, use -v or -vv for full protocol decode

listening on eth0, link-type EN10MB (Ethernet), capture size 96 bytes

20:49:24.844327 IP 192.168.0.1 > 192.168.0.2: icmp 64: echo request seq 1

20:49:24.844713 IP 192.168.0.2 > 192.168.0.1: icmp 64: echo reply seq 1

2 packets captured

2 packets received by filter

0 packets dropped by kernel

5.) Análise TCPdump utilizando o ping em um IP não existente na rede

tcpdump: verbose output suppressed, use -v or -vv for full protocol decode

listening on eth0, link-type EN10MB (Ethernet), capture size 96 bytes

20:53:35.428375 arp who-has 192.168.0.10 tell 192.168.0.1

20:53:36.428204 arp who-has 192.168.0.10 tell 192.168.0.1

20:53:37.428030 arp who-has 192.168.0.10 tell 192.168.0.1

3 packets captured

3 packets received by filter

0 packets dropped by kernel

6.) Análise TCPDump utilizando ping no broadcast 192.168.0.255

tcpdump: verbose output suppressed, use -v or -vv for full protocol decode

listening on eth0, link-type EN10MB (Ethernet), capture size 96 bytes

20:55:17.155627 IP 192.168.0.1 > 192.168.0.255: icmp 64: echo request seq 1

20:55:17.156659 arp who-has 192.168.0.1 tell 192.168.0.4

20:55:17.156695 arp reply 192.168.0.1 is-at 00:08:74:b5:c8:b7

20:55:17.157031 IP 192.168.0.4 > 192.168.0.1: icmp 64: echo reply seq 1

20:55:17.157032 arp who-has 192.168.0.1 tell 192.168.0.3

20:55:17.157096 arp reply 192.168.0.1 is-at 00:08:74:b5:c8:b7

20:55:17.157281 arp who-has 192.168.0.1 tell 192.168.0.2

20:55:17.157297 arp reply 192.168.0.1 is-at 00:08:74:b5:c8:b7

20:55:17.157405 IP 192.168.0.3 > 192.168.0.1: icmp 64: echo reply seq 1

20:55:17.157655 IP 192.168.0.2 > 192.168.0.1: icmp 64: echo reply seq 1

20:55:18.156802 IP 192.168.0.1 > 192.168.0.255: icmp 64: echo request seq 2

20:55:18.157175 IP 192.168.0.2 > 192.168.0.1: icmp 64: echo reply seq 2

20:55:18.157425 IP 192.168.0.3 > 192.168.0.1: icmp 64: echo reply seq 2

20:55:18.157549 IP 192.168.0.4 > 192.168.0.1: icmp 64: echo reply seq 2

20:55:19.157635 IP 192.168.0.1 > 192.168.0.255: icmp 64: echo request seq 3

20:55:19.157944 IP 192.168.0.2 > 192.168.0.1: icmp 64: echo reply seq 3

20:55:19.158068 IP 192.168.0.4 > 192.168.0.1: icmp 64: echo reply seq 3

20:55:19.158193 IP 192.168.0.3 > 192.168.0.1: icmp 64: echo reply seq 3

18 packets captured

18 packets received by filter

0 packets dropped by kernel